# Energy conversions, machines and efficiency

When a

##### Body

### FORCE acts on a

over a DISTANCE

then WORK is done

#### F

#### x

#### s

#### =

#### W

Force

measured in Newtons

distance

measured in meters

Work

measured in Joules

LAW OF CONSERVATION OF ENERGY :

energy cannot be created or destroyed

Machine

Energy in (work in)

W = F.s

Energy out : useful

i.e. ENERGY is transferred (passed on from one body to another) and, possibly, transformed

(changed from one form to another)

For a perfect machine (that gives out as much energy as is put into it, i.e. does as much useful work as was done on it) :

Energy in

Energy ‘wasted’ (e.g. heat and sound)

Energy out

W

#### =

#### =

W

A machine cannot give you more energy than what you put in

A machine cannot do more work than is done on it

F  s

=

F   s

A machine simply changes the form of the work done / energy available

A machine can increase force or increase distance by trading one for the other

F  s

=

F   s

Note that a machine will always ‘waste’ some of the energy put into it, so a perfect machine is
not possible. The principle of F  s trade-off still applies, though.

A machine’s efficiency is the fraction of the energy the machine receives

that the machine transfers in a usable form.

Efficiency  = Energy out (useful)    x 100

Energy in

For perfect machines this is 100%. For real machines it is less than this.